Standard Operating Procedure: Imaging

This Standard Operating Procedure (SOP) describes the proper use of the Microscopes in the Centre for Optophysiology (CfO) located in room 3214B and 3216, Medical Science Building.

- Introduction

  - The CfO houses 2 Multiphoton microscopes, 1 Stimulated Emission Depletion (STED) microscope and 1 Cytation 5 automated microscope. These can be used to image fixed samples or living cells and tissues.

- Training

  - Users must complete training with the CfO Facility Manager prior to using the equipment.

- Procedure

  - Fixed or living cells/tissue should be brought to the CfO in sealed shatterproof secondary containers (e.g., Tupperware)

  - General laboratory procedures must be followed while working in the CfO, which includes, no eating, no chewing gum, and no drinking.

  - Don PPE (lab coat, nitrile gloves).

  - Turn on microscope and secure sample in the stage/chamber according to training given by facility manager.

  - Perform imaging according to training and experimental aims.

  - Following imaging remove sample from stage and shut down microscope according to training given by facility manager.

  - Clean objectives using (a) water in the case of water immersion objectives or (b) Edmund Lens cleaner in the case of oil objectives.

  - Clean surfaces with 0.5% accelerated hydrogen peroxide solution (PREempt).

  - Return cells/tissue to home lab in sealed shatterproof secondary container, or dispose of:

    - cells mounted on glass slides in a sharps container

    - living or fixed cells in multi well plates or petri dishes at level CL1 in a Biohazard waste bag

    - living or fixed cells in multi well plates or petri dishes at level CL2 in a Biohazard waste pail

https://ehs.utoronto.ca/wp-content/uploads/2021/06/Bucket-List-Poster.pdf